参数表节选：的技术参数 NCB8-18GM50-Z4

General specifications

Switching function	Normally open (NO)

Output type	Two-wire

Rated operating distance	8 mm

Installation	flush

Output polarity	DC

Assured operating distance	0 ... 6.48 mm

Reduction factor rAl	0.4

Reduction factor rCu	0.4

Reduction factor r304	0.7

Reduction factor rBrass	0.5

Nominal ratings

Operating voltage	3.5 ... 30 V

Switching frequency	0 ... 1000 Hz

Hysteresis	typ. 5%

Reverse polarity protection	reverse polarity protected

Short-circuit protection	pulsing

Voltage drop	≤ 3.5 V

Temperature drift	± 15%

Operating current	2 ... 100 mA

Off-state current	typ. 0.8 mA

Switching state indicator	LED, yellow

Pre-fault indicator	LED, red

Stability control-switch point	0,8 sr ... 0,9 sr

Functional safety related parameters

MTTFd	1370 a

Mission Time (TM)	20 a

Diagnostic Coverage (DC)	0 %

Approvals and certificates

UL approval	cULus Listed, General Purpose

CSA approval	cCSAus Listed, General Purpose

CCC approval	CCC approval / marking not required for products rated ≤36 V

Ambient conditions

Ambient temperature	-25 ... 70 °C (-13 ... 158 °F)

Storage temperature	-25 ... 85 °C (-13 ... 185 °F)

Mechanical specifications

Connection type	cable PVC , 2 m

Core cross-section	0.34 mm2

Housing material	brass, nickel-plated

Sensing face	PBT

Housing diameter	18 mm

Degree of protection	IP67

General Notes Relating to Pepperl+Fuchs Product Information