BM系列中频感应加热设备

BM Series Medium Frequency Induction Melting Machine

整机特性：

1. 中频感应加热电源选用最先进的进口IGBT功率器件作逆变元件，采用全新的数字控制技术，具有多重调节功能与保护功能;

2.  由于采用IGBT功率器件，其频率范围更广，可以从0.5KHz至20KHz，广泛用于透热、淬火、烧结、焊接、加热、退火、保温等铸锻及热处理行业中;

3.  配合金属熔炼炉 ，适用于熔炼各种金、银、铜、铝、铅、镁、锌等有色金属，及中碳钢和各种稀有金属。

Main Features:

- Medium Frequency Induction Heating Source adopted advance import IGBT power components as inverter unit , and used full new digital control technology with multiple adjusting and protection function ;

- The frequency scope more wide (from 0.5KHz to 20KHz) due to adopted IGBT, widely used for heating, quenching, firing, welding, annealing, warming , forging and other heat treatment industry;

- Equip the Melting Furnace to apply for melting all kinds of nonferrous metal  (such as gold,

- silver, copper(Cu), lead (Pb), magnesium(Mg),zinc (Zn) etc), medium carbon steel and rare metal.

- 主要技术参数：

- Main Technical Parameters:

| 型号 Model | 输入电压 The input voltage | 输出功率 The largest output power | 振荡频率 Fluctuating frequency | 冷却水压 The pressure of cooling water | 主/分机重量 The main/ Extension weight | 进水温度 The input water temperature |
| --- | --- | --- | --- | --- | --- | --- |
| BM-35AB | 3 phase 380V+10% | 35KW | 1-20KHZ | 0.03 MPA | 43kg/50kg | <40℃ |
| BM-45AB | 3 phase 380V+10% | 45KW | 1-20KHZ | 0.03 MPA | 50kg/55kg | <40℃ |
| BM-70AB | 3 phase 380V+10% | 70KW | 1-20KHZ | 0.05 MPA | 70kg/60kg | <40℃ |
| BM-90AB | 3 phase 380V+10% | 90KW | 1-20KHZ | 0.05 MPA | 75kg/65kg | <40℃ |
| BM-110AB | 3 phase 380V+10% | 110KW | 1-20KHZ | 0.05 MPA | 80kg/70kg | <40℃ |
| BM-160AB | 3 phase 380V+10% | 160KW | 0.5-10KHZ | 0.05 MPA | 85kg/75kg | <40℃ |
| BM-260AB | 3 phase 380V+10% | 260KW | 0.5-10KHZ | 0.06 MPA | 190kg/160 | <40℃ |
| BM-360AB | 3 phase 380V+10% | 360KW | 0.5-10KHZ | 0.06 MPA | 250kg/200kg | <40℃ |