KL(C)系列工业冷水机

KL(C) Series Industrial Chiller

整机特性：

1.设备配有高质量进口压缩机，高效壳管式冷凝器，不锈钢水箱，高精度温度控制系统，过流过压保护功能。超强制冷，能效高，噪音小，操作简单；

- 2.主要用于为工业设备提供低温冷却循环水。广泛适用于机械加工与制造，电子电器，电镀设备，医药化工，激光设备，印刷等工业生产的专业冷却系统。

- Main Features:

- The Industrial Chiller has equipped exported compressor,high efficient shell tubular condenser, stainless steel tank, high precision temperature control system,over current and over voltage protection function. Super cooling, high efficiency , low noise, simple operation;

- Main used to provide cooling water for industrial equipment. Widely used in mechanical processing and manufacturing ,electrical, electroplating, chemical medicine, laser equipment, printing and other industrial production.

- 技术参数 Main Technical Parameters：

| KL(C)系列冷水机 KL(C) Series Industrial Chiller |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 型号 Model |  |  |  | KL-1C | KL-2C | KL-3C | KL-4C | KL-5C | KL-8CD | KL-10CD | KL-12CD | KL-15CD | KL-20CD | KL-25CD | KL-30CT | KL-40CT |
| 额定制冷量 Refrigerating Capacity |  | 50HZ | Btu/h | 9383 | 17742 | 30946 | 39920 | 49815 | 79840 | 99630 | 106454 | 146716 | 204037 | 232016 | 310492 | 421423 |
|  |  |  | KW | 2.75 | 5.2 | 9.07 | 11.7 | 14.6 | 23.4 | 29.2 | 31.2 | 43 | 59.8 | 68 | 91 | 123.6 |
|  |  |  | Kcal/h | 2365 | 4472 | 7800 | 10062 | 12556 | 20124 | 25112 | 26832 | 36980 | 51428 | 58480 | 78260 | 106296 |
| 输入电源 Power |  |  |  | 380V/3PHASE/50HZ |  |  |  |  |  |  |  |  |  |  |  |  |
| 压缩机功率 Compressor Power |  |  | KW | 0.91 | 1.73 | 2.67 | 3.44 | 4.3 | 6.88 | 8.6 | 9.18 | 12.98 | 17.7 | 19.96 | 27.24 | 37.56 |
| 节流器Flow Regulator |  |  |  | 毛细管/膨胀阀 Capillary /Expanding Valve |  |  |  |  |  |  | 膨胀阀 Expanding Valve |  |  |  |  |  |
| 制冷剂Refrigerant |  |  |  | R22 |  |  |  |  |  |  |  |  |  |  |  |  |
| 冷凝器 Condenser | 款式Type |  |  | 高效紫铜管串铝翅片+低噪音轴流风扇High efficient copper tube string aluminum fin + Low noise axial flow fan |  |  |  |  |  |  |  |  |  |  |  |  |
| 蒸发器 Evaporator | 形式Type |  |  | 干式水箱式/干式壳管式 Dry Tank/Dry Shell tubular |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 水箱容量 Tank Volume |  | L | 20 | 30 | 50 | 70 | 90 | 120 | 200 | 200 | 270 | 350 | 350 | 420 | 510 |
| 标准冷冻水流量 Cooling Water Flow |  |  | M3/h | 0.47 | 0.89 | 1.55 | 2.0 | 2.5 | 4.01 | 5.0 | 5.3 | 7.3 | 10.2 | 11.6 | 15.5 | 21.1 |
| 内置水泵 Water Pump |  |  | HP | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 1 | 1 | 1 | 2 | 2 | 3 | 3 | 5 |
| 外形尺寸 Dimension |  |  | L(mm) | 500 | 580 | 930 | 1140 | 1140 | 1310 | 1680 | 1680 | 1800 | 2000 | 2000 | 2200 | 2940 |
|  |  |  | W(mm) | 500 | 550 | 560 | 560 | 560 | 720 | 810 | 810 | 990 | 1130 | 1130 | 1130 | 1230 |
|  |  |  | H(mm) | 725 | 925 | 970 | 1040 | 1040 | 1270 | 1270 | 1270 | 1690 | 1720 | 1925 | 2025 | 2025 |
| 机器重量Weight |  |  | kg | 75 | 100 | 120 | 150 | 150 | 280 | 320 | 350 | 680 | 720 | 1000 | 1200 | 1500 |