Specification of Electret

Condenser Microphone

(RoHS Complince)

Customer name:

Customer Model:

LISHEN Model:   BO6027ULA42C

AIRUP               CUSTOMER APPROVEL

DESIGN:    John   2016.7.6

CHKD:    Yangjun  2016.7.6

APVD:    Vincent  2016.7.6

Tel:  +86 15118003085

Fax:  +86 339 777 12

E-Mail: 175151958@qq.com

.

- KINDS OF SPEC

1. CONSTRUCTION                                 ------------(3)

2. SPECIFICATIONS                                 ------------(3)

3. MEASUREMENT CIRCUIT                         ------------(4)

4. FREQUENCY RESPONSE ALLOWANCE RANGE     ------------(4)

5. MECHANICAL SPECIFICATION                     ------------(4)

6. RELIABILITY TEST                                ------------(5)

7. LIST AND STRUCTURE OF MATERIALS             ------------(6)

8. PACKING                                         ------------(7)

PLEASE SEND US A COPY, IF YOU APPROVE

1、CONSTRUCTION

Dimensions：Φ6.0×2.7 (Unit:mm)

2、SPECIFICATIONS                -40dB（0dB=1V/Pa）=-60dB（0dB=1V/μbar）

| No. | Item | Standard |
| --- | --- | --- |
| 1 | Sensitivity | -42dB±3dB (0dB=1V/Pa) |
| 2 | Output Impedance | 2.2KΩ |
| 3 | Directivity | Unidirectional |
| 4 | Frequency Range | 100-16,000Hz |
| 5 | Operation Voltage | DC 1.0V-10V |
| 6 | Standard Power Supply | DC3.0V |
| 7 | Min Operation Voltage | 1.0V |
| 8 | Current Consumption | ≤0.5mA |
| 9 | Signal to Noise Ratio | ≥65dB |
| 10 | Sound Pressure Level | 110dB |

3、MEASUREMENT CIRCUIT

4、FREQUENCY RESPONSE ALLOWANCE RANGE

5、MECHANICAL SPECIFICATION

| Weight | Less than：0.3g |  |
| --- | --- | --- |
| Soldering Heat Shock | To be no interfere in operation after soldering heat shock, Temperature 320℃ ± 5℃ for 3 ± 0.5 seconds. |  |
| Terminal Mechanical Strength | To be no interfering in operation after pulled the terminal with 1kg strength for 1 minute. |  |
| Absolute maximum Ratings | Operating Voltage | 10V |
|  | Storage temperature range | -25～+70℃ |
|  | Operating temperature range | -40～+85℃ |

6、RELIABILITY TEST

| Vibration Test | To be no interference in operation after vibration 10Hz to 50Hz for 1 minute full amplitude 1.52mm for 2 hour at three axises. |
| --- | --- |
| Drop Test | To be no interference in operation after dropped to concrete floor each one Time from 1 meter height at three directions in state of packing. |
| Temperature Test | - After exposure at 85℃ for 72 hours, sensitivity to be within ±3dB from initial sensitivity. (The measurement to be done after 2 hours of conditioning at 20℃) - After exposure at -40℃ for 72 hours, sensitivity to be within ±3dB from initial sensitivity. (The measurement to be done after 2 hours of conditioning at 20℃) |
| Humidity Test | After exposure at 60℃±2℃ and 90 to 95% relative humidity for 200 hours, sensitivity to be within ±3dB from initial sensitivity. (The measurement to be done after 2 hours of conditioning at 20℃) |
| Temperature Cycle Test | After exposure at -25℃ for 30 minutes, at 20℃ for 10 minutes, at 70℃ for 30 minutes, at 20℃ for 10 minutes, 5 cycles, sensitivity to be within ±3dB from initial sensitivity. (The measurement to be done after 2 hours of conditioning at 20℃) |

7、 LIST AND STRUCTURE OF MATERIALS

| No. | Part name | Material Type | Qty | Origin |
| --- | --- | --- | --- | --- |
| 1. | PCB | Glass fiber | 1 | China/jiangsu |
| 2. | FET | N-Channel Junction | 1 | Korea |
| 3. | Capacitor | 10PF 0402 | 1 | Murata |
| 4. | Capacitor | 33PF 0402 | 1 | Murata |
| 5. | Case | Al | 1 | China/hebei |
| 6. | Plastic cavity | ABS | 1 | China/shandong |
| 7. | Copper ring | Copper | 1 | China/shandong |
| 8. | Back electrets | ALSET FC-CNZ | 1 | Japan |
| 9. | Spacer | Polyester film | 1 | China/shandong |
| 10. | Diaphragm | PET | 1 | DuPont |
| 11. | FELT | Cotton decron textile | 1 | China/guangdong |
| 12. | Steel mesh | Steel | 1 | China/shandong |
| 13. | Adjust plate | Copper | 1 | China/shandong |

8.  PACKING

|  | DRAWING | QTY(PCS) | SIZE(MM) | MATERIAL | MARKING |
| --- | --- | --- | --- | --- | --- |
| PACKING |  | 100 | 61×61×4 | Paper |  |
| MIDDLE |  | 1000 | 125×64×27 | Paper |  |
| OUTER BOX |  | 30000 | 230×230×275 | Paper |  |